# Brian Meyette RV-7A

# Pilot’s Operating Handbook

N432MM

N432MM Specifications

| Exterior Dimensions |  |  |
| --- | --- | --- |
| Span | 25 ft 0 in |  |
| Length | 20 ft 4 in |  |
| Height | 7 ft 10 in |  |
| Wing Area | 121 sq ft |  |
| Weights |  |  |
| Empty Weight | 1077 - 1130 lbs |  |
| Solo Weight | 1400 lbs |  |
| Max Aerobatic Weight | 1600 lbs |  |
| Maximum Useful Load | Gross minus empty |  |
| Baggage | 100 lbs |  |
| Gross Weight | 1800 lbs |  |
| Loadings |  |  |
| Wing Loading | 14.8 lb/sq ft |  |
| Power Loading | 12.0 - 9.0 lb/hp |  |
| Powerplant/Systems |  |  |
| Engine | Eggenfellner Subaru supercharged 2.5L STi 195 HP |  |
| Propeller | MT electric Constant Speed |  |
| Oil capacity & Type | 6 qt Castrol Synthetic 5W-50 |  |
| Fuel Capacity & Type | 42 US gal 100 LL |  |

Interior Dimensions and Baggage Area

VIEWS

| N432MM Performance |  |
| --- | --- |
| Speed - Solo Weight – 1400 pounds |  |
| Top Speed | 187 kt |
| Cruise [75% @ 8000 ft] | 178 kt |
| Cruise [55% @ 8000 ft] | 161 kt |
| Stall Speed | 44 kt |
| Speed - Gross Weight – 1800 pounds |  |
| Top Speed | 185 kt |
| Cruise [75% @ 8000 ft] | 177 kt |
| Cruise [55% @ 8000 ft] | 159 kt |
| Stall Speed | 50 kt |
| Ground Performance - Solo Weight – 1400 # |  |
| Takeoff Distance | 250 ft |
| Landing Distance | 350 ft |
| Ground Performance - Gross Weight – 1800 # |  |
| Takeoff Distance | 500 ft |
| Landing Distance | 500 ft |
| Climb/Ceiling - Solo Weight – 1400 pounds |  |
| Rate of Climb | 2,450 fpm |
| Theoretical Ceiling | 24,500 ft |
| Climb/Ceiling - Gross Weight – 1800 pounds |  |
| Rate of Climb | 1,850 fpm |
| Theoretical Ceiling | 21,500 ft |
| Range |  |
| Range [75% @ 8000 ft] | 656 nm |
| Range [55% @ 8000 ft] | 804 nm |

##### Performance:

Cruise Performance at 8,000’:

| KNOTS | RPM | MAP | Fuel Flow | % Power |
| --- | --- | --- | --- | --- |
| 171 | 2450 | 23” | 10.5 GPH | 75% |
| 162 | 2350 | 22” | 9.5 GPH | 65% |
| 154 | 2250 | 21” | 8.5 GPH | 55% |

No Wind Range at 8,000’:

- All range calculations include 3 gal. for engine start, taxi, takeoff and climb.

One hour (10.5 gal.) reserve:

75% Power ………………………………………………..	434 nm

65% Power ………………………………………………..	513 nm

55% Power ………………………………………………..	591 nm

No Reserve:

75% Power ………………………………………………..	608 nm

65% Power ………………………………………………..	686 nm

55% Power ………………………………………………..	765 nm

AIRSPEED LIMITATIONS

|  | SPEED | KIAS | REMARKS |
| --- | --- | --- | --- |
| VNE | Never Exceed Speed | 200 kt | Do not exceed this speed in any operations. |
| #### VNO | Maximum Structural Cruising Speed | 168 kt | Exceed this speed only in smooth air. |
| VA | Maneuvering Speed | 123 kt | Do not make full control movements above this speed. Full elevator deflection will result in a 6g load at this speed. |
| VFE | Maximum Flap Extended Speed | 96 –20deg 87 - Full | Do not exceed this speed with flaps down |
| Vy | Best Rate of Climb | #### 96 kt |  |
| Vx | Best Angle of Climb | 70 kt |  |
| Vs | Stall Speed Clean | 55 kt |  |
| Vso | Stall Speed Landing Configuration | 50 kt |  |

AIRSPEED INDICATOR MARKINGS

| MARKING | KIAS VALUE OR RANGE | SIGNIFICANCE |
| --- | --- | --- |
| White Arc | 50-87/96 kt | Full Flap Operating Range. Lower limit is Vso. Upper limit is maximum speed with flaps extended |
| Green Arc | 55-168 kt | Normal Operating Range. Lower limit is Vs. Upper limit is maximum structural cruising speed |
| Yellow Arc | 168-200 kt | Operations must be conducted with caution and only in smooth air. |
| Red Line | 200 kt | Maximum speed for all operations |

##### AEROBATIC INFORMATION

Weight Limitation – 1600 Pounds

Recommended Entry Speeds:

Loops, Horizontal Eights 	122-165 kt

Immelman Turns 		130-165 kt

Aileron Rolls, Barrel Rolls 	104-165 kt

Snap Rolls			  70- 95 kt

Vertical Rolls 			156-165 kt

Split-S 			  87- 95 kt

##### PREFLIGHT INSPECTION

1.	CABIN

- Documentation -- Available In Airplane

- Aeronautical Charts – CURRENT & APPROPRIATE TO FLIGHT

- Seat Belt Securing Control Stick --  RELEASE

- Ignition Switch  -- OFF

- Avionics -- OFF

- Master Switch -- ON

- Engine gages – ON

- Engine tachometer –  RESET FLIGHT TIME

- Fuel Quantity -- CHECK QUANTITY

- Flaps – DOWN

- Master Switch -- OFF

2. EMPENNAGE

- Tail Tie-Down -- DISCONNECT

- Control Surfaces -- CHECK freedom of movement and security

- Static Sources (both sides of fuselage) –CHECK for blockage

- Tail and Strobe--CHECK condition

3. RIGHT WING

- Aileron -- CHECK freedom of movement and security

- Flap -- CHECK security

- Nav and Strobe--CHECK condition

- Right Landing Light -- CHECK condition

- Wing Tie-Down -- DISCONNECT

- Main Wheel Tire -- CHECK for proper inflation

- Chock -- REMOVE

- Right Wing Tank – SUMP

- Fuel Quantity -- CHECK VISUALLY

- Fuel Filler Cap – SECURE

5. NOSE

- Engine Oil Level -- CHECK, do not operate with less than 4 quarts

- Propeller and Spinner -- CHECK for nicks and security

- Cowl Hinge Pins – CHECK for security

- Air Inlet -- CHECK for restrictions

- Nose Wheel Tire -- CHECK for proper inflation

- Master sw – ON

- Fuel Boost Pump – ON

- Gascolator – SUMP

- Fuel Boost Pump – OFF

- Master sw -- OFF

- Chock -- REMOVE

- Fuel Tank Vents -- CHECK for blockage

6. LEFT WING

- Wing Tie-Down -- DISCONNECT

- Main Wheel Tire -- CHECK for proper inflation

- Chock -- REMOVE

- Left Wing Tank -- SUMP

- Fuel Quantity -- CHECK VISUALLY

- Fuel Filler Cap -- SECURE

- Pitot Tube Cover -- REMOVE and check for blockage

- Left Landing Light -- CHECK condition

- Nav and Strobe--CHECK Condition

- Aileron -- CHECK freedom of movement and security

- Flap -- CHECK security

#### BEFORE STARTING ENGINE

- Preflight Inspection -- COMPLETE

- Seat Belts and Shoulder Harnesses -- ADJUST and LOCK

- Fuel Selector Valve -- DESIRED TANK

- Avionics and Electrical -- OFF

- Brakes -- SET

- Circuit Breakers -- CHECK IN

- Canopy adjust

#### STARTING ENGINE (cold)

- Throttle – IDLE

- Master Switch-Alternator – ON

- Fuel Aux Boost Pump -- ON

- Prop -- HIGH RPM

- Flaps -- UP

- Fuel Boost Pump -- OFF

- Propeller Area -- CLEAR

- Ignition Switch -- START

- Avionics & Instruments – ON

- Oil Pressure -- CHECK   25  psi at idle

- Nav & Strobe – ON

### STARTING ENGINE (Warm)

- Mixture -- LEAN

- Throttle to 1/8

- Prop -- HIGH RPM

- Master Switch-Alternator – ON

- Flaps -- UP

- Propeller Area -- CLEAR

- Ignition Switch – START

- Mixture—Full Rich

- Avionics & Instruments -- ON

- Oil Pressure -- CHECK   25  psi at idle

- Nav & Strobe – ON

BEFORE TAKEOFF

- Brakes -- SET

- Canopy  -------- Main Latch – SECURE

- Flight Controls -- FREE and CORRECT

- Flight Instruments – SET
        	Altimeter – CORRECT PRESSURE
         D/G -- CORRECT HEADING
    		A/H -- ALIGNED 
        	GPS-- ON

- Fuel Selector Valve -- DESIRED TANK

- Mixture -- RICH (below 3000’)

- Elevator and Aileron Trim -- NEUTRAL

- Throttle -- 1700 RPM

- Prop – cycle CHECK operation

- Engine Instruments -- CHECK

- Throttle -- IDLE

- Radios -- SET

- Fuel Boost Pump -- ON

- Transponder – ALTITUDE

- Passenger – READY and willing

##### TAKEOFF

#### NORMAL TAKEOFF

- Wing Flaps – UP

- Prop – HIGH RPM

- Throttle --  FULL OPEN

- Elevator Control – LIFT NOSE WHEEL (at 56 kt)

- Climb Speed --  109 kt

#### SHORT FIELD TAKEOFF

- Wing Flaps – 10 Deg

- Prop – HIGH RPM

- Brakes – APPLY

- Throttle – FULL OPEN

- Brakes – RELEASE

- Climb Speed – 78 kt  (Vy)

##### ENROUTE CLIMB

- Airspeed – 125-150 mph

- Throttle – 25 in Hg, or full throttle

- Prop – 2500 RPM

- Fuel Pressure – CHECK

##### CRUISE

- Throttle – 23 in Hg

- Prop – 2300 RPM

- Trim – ADJUST

##### LANDING

- Approach speed 90 mph

- Flaps 20 deg.

- Prop control full rpm

- Engine 1800 rpm

- 80 mph final

- 40 deg. flaps

##### AFTER LANDING

- Wing Flaps – UP

- Boost Pump – OFF

- Transponder – STANDBY

ENGINE SHUTDOWN

- Flaps – DOWN

- Prop – FULL FORWARD

- Throttle – IDLE

- CHT decidedly dropped

- All electrical sw  – OFF

- Avionics and Instr.-- OFF

- Mixture – IDLE CUT-OFF

- Wait for shut down

- Master – OFF

#### SECURING AIRCRAFT

- Wheel Chocks

- Wing & Tail Tie-Down

- Pitot Tube Cover

- Cockpit

- Ignition Key – REMOVED

- Master and Electrical Switches – OFF

- Canopy Locked

##### WEIGHT AND BALANCE DATA

Make: Brian Meyette			Model:  RV-7A

Serial: 71008			Registration: N432MM

Maximum Weights:

Aerobatic Category …………………………	1600 Lbs

Utility Category …………………………….	1700 Lbs

Normal Category ……………………………	1800 Lbs

Datum= 70 inches forward of wing leading edge (L.E.)

Design C.G. Range = 	15% to 29% of wing chord

8.7” to 16.82” from L.E.

78.7” to 86.82” aft of Datum

Wing L.E. = 70 inches aft of datum

Main wheel right = 93.96” aft of datum

Main wheel left =   93.96” aft of datum

Nose wheel =          39.11” aft of datum

Fuel ……………………	80” aft of datum

Pilot and Passenger ……	97.48” aft of datum

Baggage ……………….	126.78” aft of datum

Aircraft weighed empty in level flight attitude.

(Includes 8 qts. of oil, no fuel)

Sample:

Weight		Arm		Moment

Aircraft

Fuel (6lbs/gal)	252		80____	20,160___

Pilot		185		97.48		18,033.80_

Passenger	170		97.48		16,571.60_

Baggage	  76		126.78_	9,635.28__

Total		683				64,400.48_

CG = Total Moment / Total Weight

CG = 	83.68	 in. aft of datum

CG Range = 78.7 to 86.82 inches aft of datum

Empty CG = 79.33      Empty Weight = 1117 lbs.

##### WEIGHT AND BALANCE DATA

*All CG ranges for possible flight conditions are within design limits

|  | Left Main | Rt. Main | Nose |  | Total |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Weight | 402 | 417 | 298 |  | 1117 |  |  |  |
| Component | Weight | * | Arm | = | Moment |  |  |  |
| Lmain | 402 |  | 93.96 |  | 37771.92 |  |  |  |
| Rmain | 417 |  | 93.96 |  | 39181.32 |  |  |  |
| Nose | 298 |  | 39.11 |  | 11654.78 |  |  |  |
| sub total | 1,117 |  |  |  | 88608.02 |  |  |  |
| CG Range | 78.7 to 86.82 in aft of datum |  |  |  |  |  |  |  |
| Empty C.G | 79.33 |  |  |  |  |  |  |  |
| Max. Gross Weight |  |  |  |  | Full Baggage, No Pax |  |  |  |
| Item | Weight | Arm | Moment |  | Item | Weight | Arm | Moment |
| Aircraft | 1,117 |  | 88,608.02 |  | Aircraft | 1,117 |  | 86,224.02 |
| Fuel (6 lbs/gal) | 252 | 80 | 20,160 |  | Fuel (6 lbs/gal) | 252 | 80 | 20,160 |
| Pilot | 190 | 97.48 | 18,521.2 |  | Pilot | 185 | 97.48 | 18,033.8 |
| Passenger | 185 | 97.48 | 18,033.8 |  | Passenger | 0 | 97.48 | 0 |
| Baggage | 56 | 126.78 | 7,099.68 |  | Baggage | 100 | 126.78 | 12,678 |
| Total | 1,800 |  | 152,422.7 |  | Total | 1,654 |  | 137,095.32 |
| CG: | 84.68 |  |  |  | CG: | 82.89 |  |  |
| Zero Fuel CG | 85.97 |  |  |  | Zero Fuel CG | 83.41 |  |  |
| Pilot, Pax, No Baggage |  |  |  |  | Pilot, No Pax, No Baggage |  |  |  |
| Item | Weight | Arm | Moment |  | Item | Weight | Arm | Moment |
| Aircraft | 1,117 |  | 86,224.02 |  | Aircraft | 1,117 |  | 86,224.02 |
| Fuel (6 lbs/gal) | 252 | 80 | 21,160 |  | Fuel (6 lbs/gal) | 252 | 80 | 21,160 |
| Pilot | 220 | 97.48 | 21,445.6 |  | Pilot | 185 | 97.48 | 18,033.8 |
| Passenger | 211 | 97.48 | 20,568.28 |  | Passenger | 0 | 97.48 | 0 |
| Baggage |  |  |  |  | Baggage |  | 126.78 |  |
| Total | 1,800 |  | 148,397.9 |  | Total | 1,554 |  | 124,417.82 |
| CG: | 82.44 |  |  |  | CG: | 80.06 |  |  |
| Zero Fuel CG | 82.84 |  |  |  | Zero Fuel CG | 80.08 |  |  |

##### Engine Information

Model: ……..	Eggenfellner Subaru STi

HP: …………	195

Fuel: ………..	91/96 or 100/130 octane minimum

………..	100LL

Oil Filter: …..	Champion CH48110

OIL:  Avg	MIL-L-6082	Ashless Dispersant

Ambiant Air	   Grades		     Grades

Above 80F	SAE 60		      SAE 60,20w50

Above 60F	SAE 60		      SAE 60

30 – 90F		SAE 40		      SAE 40,50

0-70F		SAE 30		      SAE 30,40 or SAE 40

###### Below 10F		SAE 20		      SAE 30 or 20w30

###### Oil Sump Capacity …….. 8 U.S. Quarts

###### Minimum Safe Quantity ...2 U.S. Quarts

###### Operating Conditions:

###### Oil Inlet Temp:		82 deg C desired, 118 deg C Maximum

###### Oil Pressure:		95 psi max; 	 55 psi min; 	 25 psi idle

###### Fuel Pressure:		 ? psi max;	 ? psi min;	 ? psi desired

###### Cyl. Head Temp	66 deg C – 205 deg C desired range,  260 deg C max

###### Max oil consumption .45qts/hr. at cruise

###### Conversion table:

| Celc. | Fahr. | Celc. | Fahr. | Celc. | Fahr. | Celc. | Fahr. | Celc. | Fahr. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 50 | 60 | 140 | 110 | 230 | 160 | 320 | 210 | 410 |
| 16 | 60 | 66 | 150 | 116 | 240 | 166 | 330 | 216 | 420 |
| 21 | 70 | 71 | 160 | 121 | 250 | 171 | 340 | 221 | 430 |
| 27 | 80 | 77 | 170 | 127 | 260 | 177 | 350 | 227 | 440 |
| 32 | 90 | 82 | 180 | 132 | 270 | 182 | 360 | 232 | 450 |
| 38 | 100 | 88 | 190 | 138 | 280 | 188 | 370 | 238 | 460 |
| 43 | 110 | 93 | 200 | 143 | 290 | 193 | 380 | 243 | 470 |
| 49 | 120 | 99 | 210 | 149 | 300 | 199 | 390 | 249 | 480 |
| 54 | 130 | 104 | 220 | 154 | 310 | 204 | 400 | 254 | 490 |

EMERGENCY PROCEDURES

AIRSPEEDS FOR EMERGENCY OPERATIONS

Engine Failure After Takeoff:

Wing Flaps Up …………………………	90 mph

Wing Flaps Down ……………………..	80 mph

Maneuvering Speed (Va) ……………………….	130 mph

Maximum Glide ………………………………… 	90 mph

ELECTRICAL / ALTERNATOR FAILURE

- Avionics –OFF

- Master Switch – OFF

- Alt Field -- OFF

- Master Switch – ON

IF ALTERNATOR IS STILL OFF-LINE:

- Master Switch – ON

- Electrical Switches – OFF

- Alternator Field – OFF

- Avionics – ON as required

- Electrical Equipment – ON, as required

- Flight – TERMINATE as soon as practical,  aircraft is on battery reserves only.

EMERGENCY PROCEDURES

ENGINE FAILURES

ENGINE FAILURE DURING TAKEOFF RUN

- Throttle –IDLE

- Brakes – APPLY

- Wing Flaps – RETRACT

- Mixture – IDLE CUT-OFF

- Ignition Switch – OFF

- Master Switch – OFF

ENGINE FAILURE IMMEDIATELY AFTER TAKEOFF

- Airspeed – 80 mph

- Mixture – IDLE CUT-OFF

- Fuel Selector Valve – OFF

- Ignition Switch – OFF

- Wing Flaps – AS REQUIRED

- Master Switch – OFF

ENGINE FAILURE DURING FLIGHT

- Airspeed – 90 mph

- Boost Pump – ON

- Fuel Selector – SWITCH TANKS

- Mixture – RICH

- Ignition Switch – BOTH, LEFT, RIGHT

- Transponder – 7700

EMERGENCY PROCEDURES

FIRES

DURING START ON GROUND

- Cranking – CONTINUE, to get a start which would suck the flames and accumulated fuel through the carb and into the engine.

If engine starts:

- Power – 1700 RPM for a few minutes

- Engine – SHUTDOWN and inspect for damage

If engine fails to start:

- Throttle – FULL OPEN

- Mixture – IDLE CUT-OFF

- Cranking – CONTINUE

- Fire Extinguisher – OBTAIN

- Engine – SECURE

ENGINE FIRE IN FLIGHT

- Mixture – IDLE CUT-OFF

- Fuel Selector Valve – OFF

- Master Switch – OFF

- Cabin Heat and Air – OFF

ELECTRICAL FIRE IN FLIGHT

- Master Switch – OFF

- Avionics – OFF

- All Other Switches (except ignition) – OFF

- Vents/ Cabin Air/ Heat – CLOSED

- Fire Extinguisher – ACTIVATE (if available)

CABIN FIRE

- Master Switch – OFF

- Vents/ Cabin Heat – CLOSED

- Fire Extinguisher – ACTIVATE (if available)

WING FIRE

- Nav & Strobe Lights – OFF

- Landing Light – OFF

FAR 91.125 - ATC light signals.

ATC light signals have the meaning shown in the following table:

| Color and type of signal | Meaning with respect to aircraft on the surface | Meaning with respect to aircraft in flight |
| --- | --- | --- |
| Steady green | Cleared for takeoff | Cleared to land |
| Flashing green | Cleared to taxi | Return for landing |
| Steady red | Stop | Give way to other aircraft and continue circling. |
| Flashing red | Taxi clear of runway in use | Airport unsafe—do not use |
| Flashing white | Return to starting point on airport | N/A |
| Alternating red and green | Exercise extreme caution | Exercise extreme caution |

## Compass Headings, VFR under 18,000ft

| Course | Altitude |
| --- | --- |
| 0 – 179 degrees | Odd thousand +500 |
| 180 – 360 degrees | Even thousand + 500 |